#ALU xuất kết quả ra led 7 đoạn

#sw17 – reset

#sw [16:15] – chọn mode: 00_add, 01_sub, 10_compare_max, 11_compare_min

#sw[13:7] – operand_B

#sw[6:0] – operand_A

# ===================== KHỞI TẠO BẢNG MÃ 7-SEG =====================

addi x1, x0, 0x40     # 0

sw   x1, 0(x0)

addi x1, x0, 0x79     # 1

sw   x1, 4(x0)

addi x1, x0, 0x24     # 2

sw   x1, 8(x0)

addi x1, x0, 0x30     # 3

sw   x1, 12(x0)

addi x1, x0, 0x19     # 4

sw   x1, 16(x0)

addi x1, x0, 0x12     # 5

sw   x1, 20(x0)

addi x1, x0, 0x02     # 6

sw   x1, 24(x0)

addi x1, x0, 0x78     # 7

sw   x1, 28(x0)

addi x1, x0, 0x00     # 8

sw   x1, 32(x0)

addi x1, x0, 0x10     # 9

sw   x1, 36(x0)

# ===================== KHỞI TẠO THANH GHI =====================

addi x1, x0, 1

addi x2, x0, 2

addi x3, x0, 3

addi x4, x0, 4

addi x5, x0, 5

addi x6, x0, 6

addi x7, x0, 7

addi x8, x0, 8

addi x9, x0, 9

#===================================================================

lui x29, 0x10010 			#lấy địa chỉ sw

lui x30, 0x10000 			# lấy địa chỉ ledr

lui x31, 0x10002 			# lấy địa chỉ 7led

main:

lw x11, 0(x29)				#x11 chứa giá trị sw

sw x11, 0(x30)			#xuất ra ledr

andi x12, x11,0x7F			#x12 chứ giá trị A [6:0]

srli x13, x11, 7

andi x13, x13, 0x7F			#x13 chứ giá trị B [14:7]

srli x14, x11, 15

andi x14, x14, 3			#x14 chọn mode [16:15]

srli x24, x11, 14

andi x24, x24, 1			#x24 chứa bit dấu

# ==== SIGN EXTEND nếu ở chế độ signed ====

beq x24, x0, skip_sign_ext

# sign-extend A (7-bit)

slli  x12, x12, 25                  		# shift left (32-7)=25

srai  x12, x12, 25                  		# arithmetic shift right -> sign extend

# sign-extend B (7-bit)

slli  x13, x13, 25

srai  x13, x13, 25

skip_sign_ext:

#compare mode case

beq x14, x0, do_add 			#mode_add_00

addi x15, x0, 1

beq x14, x15, do_sub		#mode_sub_01

addi x15, x0, 2

beq x14, x15, do_max		#mode-compare_max _10

jal x0, do_min			#mode-compare_min_11

do_add:

add x16, x12, x13		#x16=A + B

jal x0, write_result

do_sub:

sub x16, x12, x13		#x16=A – B

jal x0, write_result

do_max:

#sltu x17, x12, x13		#x17 = (A < B) unsigned ?

beq x24, x0, compare_unsigned_max

slt  x17, x12, x13      		# signed compare

jal  x0, compare_done_max

compare_unsigned_max:

sltu x17, x12, x13      		# unsigned compare

compare_done_max:

bne x17, x0, choose_B	# if A < B -> choose B

add x16, x12, x0		#result = A

jal x0, write_result

choose_B:

add x16, x13, x0		#result = B

jal x0, write_result

do_min:

#sltu x17, x12, x13		#x17 = (A < B) unsigned ?

beq x24, x0, compare_unsigned_min

slt  x17, x12, x13      		# signed compare

jal  x0, compare_done_min

compare_unsigned_min:

sltu x17, x12, x13      		# unsigned compare

compare_done_min:

beq x17, x0, choose_B_min	# if A >= B -> choose B min

add x16, x12, x0		#result = A

jal x0, write_result

choose_B_min:

add x16, x13, x0		#result = B

jal x0, write_result

#========================================================================

write_result:

# ---- kiểm tra nếu kết quả âm ----

addi x26, x0, 0       		# x26 = cờ âm (0: dương, 1: âm)

blt x16, x0, make_positive 	# nếu x16 < 0 thì chuyển sang dương

beq  x0, x0, continue_div

make_positive:

sub x16, x0, x16       		# x16 = -x16 (lấy trị tuyệt đối)

addi x26, x0, 1        		# cờ âm = 1

continue_div:

addi x17, x0, 100

addi x18, x0, 0          		# Hàng trăm

div_hundred:

blt  x16, x17, done_hundred

sub  x16, x16, x17

addi x18, x18, 1

beq  x0, x0, div_hundred

done_hundred:

addi x17, x0, 10

addi x19, x0, 0          		# Hàng chục

div_ten:

blt  x16, x17, done_ten

sub  x16, x16, x17

addi x19, x19, 1

beq  x0, x0, div_ten

done_ten:

addi x20, x16, 0         		# Hàng đơn vị

#18_hàng_trăm, 19_hàng_chục, 20_hàng_đơn_vị

# ===================== GIẢI MÃ HÀNG ĐƠN VỊ =====================

beq x20, x0, so_0_dv

beq x20, x1, so_1_dv

beq x20, x2, so_2_dv

beq x20, x3, so_3_dv

beq x20, x4, so_4_dv

beq x20, x5, so_5_dv

beq x20, x6, so_6_dv

beq x20, x7, so_7_dv

beq x20, x8, so_8_dv

beq x20, x9, so_9_dv

# ----------------- HÀNG CHỤC -----------------

hang_chuc:

beq x19, x0, so_0_chuc

beq x19, x1, so_1_chuc

beq x19, x2, so_2_chuc

beq x19, x3, so_3_chuc

beq x19, x4, so_4_chuc

beq x19, x5, so_5_chuc

beq x19, x6, so_6_chuc

beq x19, x7, so_7_chuc

beq x19, x8, so_8_chuc

beq x19, x9, so_9_chuc

# ----------------- HÀNG TRĂM -----------------

hang_tram:

beq x18, x0, so_0_tram

beq x18, x1, so_1_tram

beq x18, x2, so_2_tram

beq x18, x3, so_3_tram

beq x18, x4, so_4_tram

beq x18, x5, so_5_tram

beq x18, x6, so_6_tram

beq x18, x7, so_7_tram

beq x18, x8, so_8_tram

beq x18, x9, so_9_tram

# ===================== BẢNG GIẢI MÃ 7-SEG =====================

# ----- Hàng đơn vị -----

so_0_dv: lw x21, 0(x0);    jal x0, hang_chuc

so_1_dv: lw x21, 4(x0);    jal x0, hang_chuc

so_2_dv: lw x21, 8(x0);    jal x0, hang_chuc

so_3_dv: lw x21, 12(x0);   jal x0, hang_chuc

so_4_dv: lw x21, 16(x0);   jal x0, hang_chuc

so_5_dv: lw x21, 20(x0);   jal x0, hang_chuc

so_6_dv: lw x21, 24(x0);   jal x0, hang_chuc

so_7_dv: lw x21, 28(x0);   jal x0, hang_chuc

so_8_dv: lw x21, 32(x0);   jal x0, hang_chuc

so_9_dv: lw x21, 36(x0);   jal x0, hang_chuc

# ----- Hàng chục -----

so_0_chuc: lw x22, 0(x0);   jal x0, hang_tram

so_1_chuc: lw x22, 4(x0);   jal x0, hang_tram

so_2_chuc: lw x22, 8(x0);   jal x0, hang_tram

so_3_chuc: lw x22, 12(x0);  jal x0, hang_tram

so_4_chuc: lw x22, 16(x0);  jal x0, hang_tram

so_5_chuc: lw x22, 20(x0);  jal x0, hang_tram

so_6_chuc: lw x22, 24(x0);  jal x0, hang_tram

so_7_chuc: lw x22, 28(x0);  jal x0, hang_tram

so_8_chuc: lw x22, 32(x0);  jal x0, hang_tram

so_9_chuc: lw x22, 36(x0);  jal x0, hang_tram

# ----- Hàng trăm -----

so_0_tram: lw x23, 0(x0);   jal x0, hien_so

so_1_tram: lw x23, 4(x0);   jal x0, hien_so

so_2_tram: lw x23, 8(x0);   jal x0, hien_so

so_3_tram: lw x23, 12(x0);  jal x0, hien_so

so_4_tram: lw x23, 16(x0);  jal x0, hien_so

so_5_tram: lw x23, 20(x0);  jal x0, hien_so

so_6_tram: lw x23, 24(x0);  jal x0, hien_so

so_7_tram: lw x23, 28(x0);  jal x0, hien_so

so_8_tram: lw x23, 32(x0);  jal x0, hien_so

so_9_tram: lw x23, 36(x0);  jal x0, hien_so

# ===================== HIỂN THỊ 3 CHỮ SỐ =====================

hien_so:

slli x22, x22, 8         			# Dịch trái 8 bit (hàng chục)

slli x23, x23, 16        			# Dịch trái 16 bit (hàng trăm)

or   x22, x22, x21       		# Ghép đơn vị + chục

or   x23, x23, x22       		# Ghép thêm hàng trăm

# ---- nếu có dấu âm thì bật dấu "-" ở LED hàng trăm ----

beq x26, x0, skip_negative_led

# Mã 7-seg cho dấu "-" là 0x7F (chỉ hiển thị thanh ngang giữa)

addi x27, x0, 0x3F

slli x27, x27, 24      			# dịch vào vị trí dấu “-”

or x23, x23, x27

skip_negative_led:

sw   x23, 0(x31)         		# Ghi ra LED7

beq  x0, x0, main        		# Lặp lại

=========================================================================

04000093

00102023

07900093

00102223

02400093

00102423

03000093

00102623

01900093

00102823

01200093

00102a23

00200093

00102c23

07800093

00102e23

00000093

02102023

01000093

02102223

00100093

00200113

00300193

00400213

00500293

00600313

00700393

00800413

00900493

10010eb7

10000f37

10002fb7

000ea583

00bf2023

07f5f613

0075d693

07f6f693

00f5d713

00377713

00e5dc13

001c7c13

000c0a63

01961613

41965613

01969693

4196d693

00070c63

00100793

00f70c63

00200793

00f70c63

0380006f

00d60833

0540006f

40d60833

04c0006f

000c0663

00d628b3

0080006f

00d638b3

00089663

00060833

0300006f

00068833

0280006f

000c0663

00d628b3

0080006f

00d638b3

00088663

00060833

00c0006f

00068833

0040006f

00000d13

00084463

00000663

41000833

00100d13

06400893

00000913

01184863

41180833

00190913

fe000ae3

00a00893

00000993

01184863

41180833

00198993

fe000ae3

00080a13

060a0c63

061a0e63

082a0063

083a0263

084a0463

085a0663

086a0863

087a0a63

088a0c63

089a0e63

0a098063

0a198263

0a298463

0a398663

0a498863

0a598a63

0a698c63

0a798e63

0c898063

0c998263

0c090463

0c190663

0c290863

0c390a63

0c490c63

0c590e63

0e690063

0e790263

0e890463

0e990663

00002a83

fadff06f

00402a83

fa5ff06f

00802a83

f9dff06f

00c02a83

f95ff06f

01002a83

f8dff06f

01402a83

f85ff06f

01802a83

f7dff06f

01c02a83

f75ff06f

02002a83

f6dff06f

02402a83

f65ff06f

00002b03

f85ff06f

00402b03

f7dff06f

00802b03

f75ff06f

00c02b03

f6dff06f

01002b03

f65ff06f

01402b03

f5dff06f

01802b03

f55ff06f

01c02b03

f4dff06f

02002b03

f45ff06f

02402b03

f3dff06f

00002b83

04c0006f

00402b83

0440006f

00802b83

03c0006f

00c02b83

0340006f

01002b83

02c0006f

01402b83

0240006f

01802b83

01c0006f

01c02b83

0140006f

02002b83

00c0006f

02402b83

0040006f

008b1b13

010b9b93

015b6b33

016bebb3

000d0863

03f00d93

018d9d93

01bbebb3

017fa023

d80002e3